OEHHA Guideline Changes

Goal: Improve Protection of Children’s Health

Major Changes:

- Age Sensitivity Factors

  - Ages 0-2:	10X

  - Ages 2-16:	3X

  - Ages 16+:	1X

- Age-Specific Exposure Variable

  - Age Groups

3rd trimester – birth	birth < 2 years	2 < 9 years

2 < 16 years			16 < 30 years	16 < 70 years

  - Derived Approach for Multi-Pathway Pollutants

  - 95/80 Breathing Rate Policy for Inhalation Only Pollutants

- Fraction of Time at Home

  - Default: 0.73 – 1.0

  - Refined: 0.73 – 0.85

- Exposure Duration

  - Resident: Reduced from 70 years to 30 years

  - Worker: Reduced from 40 years to 25 years

- Spatial Averaging of Exposure Concentrations

  - Use a 400 m2 grid with 5 meter intervals

Impacts Compared to Current District Procedures:

- Residential Cancer Risk

  - 40% Increase for Inhalation Only Pollutants

  - 90% - 5X Increase for Multi-Pathway Pollutants

- Worker Cancer Risk

  - Essentially No Change

- Non-Cancer Impacts

  - New TAC: caprolactam

  - REL Changes / 8-hour RELs

benzene, 1,3 butadiene, nickel, selenium

acetaldehyde, acrolein, arsenic, formaldehyde, manganese, mercury

(The following applies to residential cancer risk)

District Implementation of OEHHA Guideline Changes

- Revisions to Regulation 2, Rule 5: New Source Review of Toxic Air Contaminants (TACs)

  - Proposed Revisions

    - Update District HRA Guidelines

    - Revise Trigger Levels

    - Add Exemption for Small Engines (< 50 bhp)

    - Revise Calculation Procedure for Certain Modified Sources

  - Open Houses Held in February

  - Board Hearing Scheduled for September 21, 2016

- New Policy or Rule for Existing Facilities

  - Concept Stage

    - Apply to Sites with Cancer Risk > 10.0 in a million

    - Require TBACT for All Significant Sources

    - No Specific Risk Caps / Retain Current Thresholds

  - Workshops in 2017